MOSAIC 1.0 - multitasking for Borland C

Rich Leggitt

Left Coast Logic

742C Nobel Drive

Santa Cruz, CA 95060

rleggitt@ix.netcom.com

First, a brief disclaimer:  Mosaic is NOT a way to multi-task DOS

applications, or run other programs in conjunction with your C

program.  This type of multitasking is a function of the operating

system, and, alas, MS-DOS (and more to the point, ROM BIOS) simply

does not have this capability.

Mosaic is a way to perform co-operative multi-tasking within a C

application.  This document assumes that the reader has a basic

familiarity with the concepts involved with multi-tasking.  However,

some of the more arcane concepts will be discussed.

In non-cooperative, or pre-emptive, scheduling system, each task is

alloted a given amount of time in which to execute.  When the time

expires (or if some higher priority event occurs), the task is

pre-empted and another task is allowed to run.  This type of

scheduling is most useful when the tasks are not cognizant of each

other (for instance, DesqView is a pre-emptive multitasking system).

However, it can be difficult to manage and is fairly slow.

A far better approach for stand-alone programs which require

multi-tasking is the use of a co-operative scheduler.  The implication

is that the tasks must co-operate to get the job done.  To do this,

they must each relinquish control of the CPU if they have nothing to

do, or if they are in the midst of a low priority, but time consuming,

task.

In co-operative round-robin scheduling systems, each task gets control

in sequence, performs whatever actions are required of it, then passes

control to the next task.  A problem with round-robin scheduling is

that tasks which are waiting for some event to occur are still

required to execute long enough to check some flag, etc., in order to

determine whether they can continue further.

In mosaic scheduler systems, on the other hand, tasks which are

waiting for resources, or events generated by other tasks or by

interrupt service routines, do not consume ANY computer time.  The

order of task execution is not predictable; tasks are 'fit together'

as required, hence the term 'mosaic' when refering to schedulers of

this type.

WHAT IS IT?

MOSAIC provides a set of tools to create and manipulate semaphores.  A

semaphore is a data structure which heads a linked list of tasks

waiting for execution.  The type SEMAPHORE is defined by MOSAIC, as

is the procedure SEMINIT which is used to initialize it before it can

be used.

A semaphore can be created for two discrete reasons:

1) Resource arbitration.  Let's say for example, that you have two

tasks:  task A, which uses the printer to occasionally print a line of

status information, and task B, which prints characters as you type

them in (see the example file MOSEX.C).  It would not do, however,

to have your typed in characters appear in the middle of the status

line.  So you create a semaphore, called PRINTER for example, to

arbitrate the usage of the printer.  When task A wishes to use the

printer, it WAITs on PRINTER.  If the printer is not in use, then task

A will be allowed to continue.  However, if task B is using the

printer, then task A will be pended.  When task B is done with the

printer, it SIGNALs the semaphore, which 'unpends' task A.

How does task A know that task B was using the printer?  Because task

B is also required to WAIT on the semaphore before using it.  The WAIT

procedure essentially marks the semaphore as busy when a task WAITs on

it, and the SIGNAL procedure marks a semaphore as idle when a task

SIGNALs it.  The fact that all tasks must WAIT on a resource before

using it, a SIGNAL the resource when done is the essence of

co-operative multi-tasking.

2) The second reason to use a semaphore is task synchronization.  For

example, let's say you again have two tasks:  task C, which reads keys

from the keyboard, and task B, which prints the entered keys on the

printer (from the above example).  Obviously, task B cannot print a

key which has not yet been typed.  Therefore a semaphore, for example

KEYPRESS, is created to synchronize task B to task C.  In this

circumstance, task B waits on the KEYPRESS semaphore.  Whenever task C

detects that a key has been pressed, it places the key in a buffer and

SIGNALs the KEYPRESS semaphore.  Task B is thus awakened, and prints

the character, then goes back to waiting on the KEYPRESS semaphore.

Aha, you say, but what if task B is required to wait for the printer

because task A is using it?  What happens if multiple keys are

pressed?  Won't task B print only the first key?  This would be true

if a semaphore was simply a bit flag, true if signalled and false if

not signalled.  However, this is not the case.  MOSAIC semaphores are

counting semaphores.  Excess SIGNALs (up to 65,535) are stored, so

that any task that WAITs on it will be allowed to proceed normally.  A

task is forced to wait only if the semaphore has no pending signals.

The big difference between an arbitration semaphore and a

synchronization semaphore, then, is that arbitration semaphores are

initially SIGNALled during initialization (assuming that the resource

is initially available, as would be the case with PRINTER).

WHERE'S THE SCHEDULER?

Strictly speaking, there is no place you can point to and say "This is

the code responsible for scheduling tasks". This is because each task

schedules itself, via the WAIT procedure, and schedules other

tasks via the SIGNAL procedure.

Whenever a task WAITs on a semaphore, if the semaphore is not

available (i.e. it's count is 0), the WAIT procedure links the task to

a linked list of tasks connect to that semaphore. It then proceeds to

awaken the first task connected to a special system semaphore called

the ready list. This semaphore, which is a list of tasks waiting to

use the CPU, is never directly signalled or waited on.

If the semaphore IS available (i.e. it's count is non-zero), the WAIT

procedure decrements the semaphore count and returns.

When a task SIGNALs a semaphore, the SIGNAL procedure unlinks the

first task from the semaphore's linked list and attaches it to the end

of the ready list. If there are were tasks waiting on the semaphore,

the SIGNAL increments the semaphore count. In any event SIGNAL always

returns to the caller. Because of this, it is acceptable to SIGNAL

semaphores from interrupt service routines.

It is important to understand that a task that calls WAIT can never be

certain whether it was actually pended or not, nor should it care. If

the resource it is waiting for is unavailable, it will be pended. When

the resource becomes available, the task will be restarted from the

exact spot where it left off, i.e. the first instruction following the

call to WAIT.

Another procedure, called PAUSE, checks the ready list. If the ready

list is empty, PAUSE simply returns. If the ready list has a task

waiting to run, however, PAUSE pends the current task and starts the

waiting task. The PAUSEd task will run when the previously pended

tasks have all either WAITed or PAUSEd.

Let's take, for example, the task that reads keys from the keyboard.

This task could be implemented in one of three ways:

1) It could be an interrupt 9 interrupt service handler. Since

interrupt services routines can use the SIGNAL procedure, it could

still signal the KEYPRESS semaphore whenever a key is pressed.

2) It could wait on yet another syncronization semaphore signalled

by an interrupt 9 ISR.

3) It could simply wait for the kbhit() function to

return true, via the following line of code:

while (!kbhit()) pause();

The task runs fairly often, checks for a keypress, and if none is

available, pauses and tries again.

The first two methods are the most CPU efficient, but the last is

the easiest to implement.

MULTITASKING AND THE PC.

One reason for multitasking is to make efficient use of the CPU during

'I/O bound' operations.  This is a term used to describe a program or

process which is constrained by the speed at which an external device

can respond to its instructions.

For example, let's assume that you wish to write something to the

disk (at a very low level).  First, you must instruct the disk

controller to seek to the correct track:  this takes time.  When the

seek is complete, you instruct the controller to DMA a block of memory

and write it to the disk at a given sector:  again, this takes a

finite amount of time.  For the purposes of this discussion, this can

be called an I/O bound process, for if you didn't have to wait for the

disk to seek and for the data to be written to the disk, you could

perform the 'write to disk' operation very quickly.

In a true multi-tasking system, the CPU would not be idle during these

wait times.  When the task had instructed the disk controller to seek

(for example), it would the wait on a semaphore to be signalled by the

disk controller when the seek was complete.  This way, any other task

with something to do could run while the disk was seeking, and the

system would be used very efficiently indeed.

Unfortunately, PC applications are practically limited by the fact

that they must use MS-DOS to write to the disk, which in turn uses ROM

BIOS. For the most part, the ROM BIOS 'write to disk' function does

not return until the function is complete; thus, the possibility of

recovering the wait time is lost. (Note I said 'for the most part'. AT

ROM BIOS does, in fact, generate an interrupt when the disk write has

been initiated, so it is theoretically possible to regain control

during write to disk. However, I haven't yet found a clean way of

supporting these functions without having to bypass MS-DOS.)

Most of the ROM BIOS hardware support functions exhibit this one-way

behavior.  In some cases it is not a problem (i.e.  serial port access

is typically done without even using ROM BIOS), and in other cases it

cannot be avoided.

MULTITASKING AND THE PROGRAMMER

The other reason for using multitasking is because, in many cases, it

greatly simplifies programming. This is especially true of unrelated

tasks. Using the example program we have developed so far, suppose

task D is responsible for keeping track of the elapsed time and

signalling task A at some time interval (task A, remember, is

responsible for printing a status line). Imagine trying to write a

standard loop program that tracks the time, waits for the key, drives

the printer, etc. Sure, it can be done, but now go look at the example

program. It is far simpler than an equivalent 'in-line'

implementation, and runs more smoothly to boot.

WHAT'S IN IT?

The following symbols are defined in the interface section of

MOSAIC.H:

typedef struct semaphore

This is used to define a variable of type semaphore.

void _start(void *task, char *stack, int size);

This is used to initialize a task.  Note that a task must be

defined as a parameterless function, and that it must NEVER exit

or attempt to return to the main procedure, or the system will

instantly crash.

*task is the address of the task to initialize, *stack is the

address of the stack buffer (defined as a local varaiable in main()), and

size is the size of the stack buffer. The macro 'task' assumes that the

name of the stack for 'task' is 'task_stack'.  Figuring the exact

amount of stack to use can be tricky.  I generally use 1024, but

for large local data structures or deeply nested routines, this may

be insufficient.  The actual allocated amount is STACK+10.  If

there is not enough heap to start the task, a heap overflow error

will occur.

Since the task stacks are defined as local variables on main()'s

stack, function main() gets all the left over stack.  For this

reason, main() is normally used for the most stack intensive

operations, such as user interface, etc.

void _wait(void)

If the ready list is not empty (i.e., other tasks are ready to

run), this will temporarily pend the current task and let the

others go.  It is functionally the equivalent of

IF WAITING(READY_LIST) THEN WAIT(READY_LIST);

PROCEDURE WAIT(VAR S:SEMAPHORE);

The current task will wait for the semaphore to be signalled

(become available).  Note that the semaphore may already be

available (signal count > 0), otherwise the task will be pended.

PROCEDURE SIGNAL(VAR S:SEMAPHORE);

This is used to inform other tasks that you are done with the

semaphore.  If no task is waiting for the semaphore, this will

increment the semaphore's signal count instead. SIGNAL does not

affect the current task.

It is legal (indeed encouraged) to signal a semaphore from within

an interrupt procedure. Where SIGNAL cannot be re-entrant, it

disables the interrupts.

FUNCTION PENDING(S:SEMAPHORE):BOOLEAN;

This returns true if the semaphore has pending signals (i.e.

semaphore signal count > 0).

FUNCTION WAITING(S:SEMAPHORE):BOOLEAN;

This returns true if any task is waiting on the semaphore.

PROCEDURE SEMINIT(VAR S:SEMAPHORE);

This must be called once to initialize semaphores.

THINGS TO WATCH FOR:

Semaphores which are used to arbitrate resources should be initially

available. Therefore, they should be signalled once as part of their

initialization.

Don't ever, ever exit or otherwise return from a task, or you will

certainly crash. Since the task has it's own stack, there is no return

address for it to return to.

It is possible to create a semaphore on a task's stack, as a local

parameter.  However, since semaphores are for inter-task

communication, it is not good for much since no other task can wait on

or signal it.

If you wish to permanently halt a task, create a semaphore called

FOREVER and have it wait on that.  The task will never run again (as

long as you don't signal FOREVER), and will not consume one more

pico-second of CPU time.  Sorry, the stack space cannot be

released.

(As a comment to the item above, it is my contention that if you need

to kill a task, you probably shouldn't have created it in the first

place. Go back and rethink your task structure.)

Don't PAUSE or WAIT from within an interrupt service routine.  SIGNAL

is OK, though.

You don't need to create an arbitration semaphore if you don't plan to

have any task WAIT or PAUSE while using that resource.

Remember, the current task owns the machine until it WAITs or PAUSEs.

There are no exceptions to this:  it cannot be pre-empted.

FINALLY...

MOSAIC is based on embedded operating system concepts that I have been

developing over a period of many years.  This is my second port

of MOSAIC to a high level language.

MOSAIC is 100% public domain, use it as you see fit. There is no

licensing, etc., etc.

However, it does contain and compile a copyright notice.  This is to

ensure that I will not find MOSAIC.TPU for sale anywhere (anyone

wishing to market MOSAIC itself, please contact me by mail).  In

return for this free unit, I ask only that you do not modify the

copyright notice (a cost of 134 bytes).

TANSTAAFL:  I will take no responsibilty for any losses, damages,

bugs, failures, errors, disasters, deaths (god forbid) or any other

problems incurred by using MOSAIC.  This is not to say that I haven't

thoroughly tested MOSAIC (I have), or that I don't think that MOSAIC

will work just fine (I do).

It is my sincerest hope that you find MOSAIC to be useful, and that it

will simplify your programming effort.

Rich Leggitt - September 1990